# PRODUCTION READINESS REVIEW			 PRR-HEC-Elec-B1

# COLD FRONT-END ELECTRONICS

(Revision Nov. 2000)

# Design of the HEC PSB Boards

Max-Planck-Institut für Physik, München

W. D. Cwienk, H. Brettel, J. Fent

PSB FRONT-END BOARD

# PSB FRONT-END BOARD                                                 HEC-97 PRR

## List of Contents

## Introduction

## Principle

## Boards (description)

## LV System of HEC

Principles

Power consumption of  PSB boards (table)

## Schematics of boards

1	PSB 1		(LSEG 1)	signal connections

2	PSB 2_1	(LSEG 2)	signal connections

3	PSB 2_2	(LSEG 2)	signal connections

4	PSB 3		(LSEG 3)	signal connections

5	PSB 4		(LSEG 3)	signal connections

6	HEC PSB_1			layout (component layer)

7	HEC PSB_2_1		layout (component layer)

8	HEC PSB_2_2		layout (component layer)

9	HEC PSB_3			layout (component layer)

10	HEC PSB_4			layout (component layer)

## Layer definition

## Schematic partial (1 chip group)

# PSB FRONT-END BOARD                                                 HEC-97 PRR

## Introduction

The cold front-end electronics of a hadronic end cap, based on a GaAs preamplifier and summing ASICs (MPI-BB96B), is contained in 160 PSB front-end  boards (320 boards for both end cap calorimeters), mounted at the circumference of the calorimeter wheels. There are 5 types of boards, different in their mechanical dimensions and the number of chips, but equal in the basic layout of the electronic circuits. The mean number of chips per board is 14.

This paper contains all details about the PSB boards and it’s passive components. Information about the LV- supply system is included too. The ASICS are treated separately in the paper „BB96B FRONT-END CHIP“.

An ASIC contains 8 preamplifiers and two drivers. All active and passive components  are on the chip except the feedback combination of the drivers. A small number of  resistors and capacitors on the boards isolate the signals and filter the supply voltages.

The PSBs and the chips are designed in such a way, that problems in one channel should not effect the overall system. Each board has a it’s own power supply.

The quality assurance procedure is described in the paper „QUALITY ASSURANCE“.

# PSB FRONT-END BOARDS

## Principle:

For each j-segment of the wheel there are 4 longitudinal segments.

On each longitudinal segment active electronics units are mounted,

called PSBs (Preamplifier and Summing Boards).

(See schematics 1 to 5.)

LSEG 1 has 1 unit,

PSB_1 with 112 inputs into 14 BB96-chips and summing into 24 outputs.

LSEG 2 has 2 units,

PSB_2_1 with 88 inputs into 12 BB96-chips and summing into 8 outputs.

PSB_2_2 with 120 inputs into 15 BB96-chips and summing into 15 outputs.

LSEG 3 has 1 unit,

PSB_3 with 96 inputs into 14 BB96-chips and summing into 21 outputs.

LSEG 4 has 1 unit,

PSB_4 with 84 inputs into 12 BB96-chips and summing into 22 outputs.

## Boards

The printed circuit boards which carry the GaAs-chips and the passive components

are of standard material FR4 and consist of 8 layers. (See figure ....)

The above listed 5 types of PSB-units are all different (number of chips and

number of summed input signals), but only 3 different layouts of printed circuit boards

are needed:

| Type of printed circuit board | Type of PSB-unit |
| --- | --- |
| PSB1x3x4 | PSB_1, PSB_3, PSB_4 |
| PSB_2_1 | PSB_2_1 |
| PSB_2_2 | PSB_2_2 |

## Active Components

GaAs-ICs type BB96B, design MPI, production TriQuint

## Passive Components

SMD resistors, size 0805, TK 100

Ceramic chip capacitors, size 0805 and 1206, NP0

Ceramic chip capacitors, size 1206, X7R

# LV System for ATLAS-HEC Calorimeter

There is a separate power distribution for each PSB. The PSBs are supplied with 3 voltages:

+7, +4 and -3V.

The power supplies are floating according to the ATLAS grounding rules.

There is a supply and a return line for every voltage,  6 lines for a PSB.

The low voltage power for one quadrant, delivered by a supply box between two tile calorimeter fingers, is distributed on 2 boards inside the front-end crate and then passes into

the cryostat.

As a j-segment has 5 PSBs and a quadrant consists of 8 segments, the total number of

low voltage lines per feed-through is 240.

As there is no possibility to measure the voltages at the boards, they have to be calculated from

the measured currents and the line resistances and then adjusted by the regulators.

The power dissipation is different at warm and cold operation.

Schematic 1

Schematic 2

Schematic 3

Schematic 4

Schematic 5

Fig. 1

Fig. 2

Fig. 3

Fig. 4

Fig. 5

Fig. 6

Table 1

| 03. Sep 98 |  | Power - Consumption BB96B |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PAS | WARM | mA | mW | mA | mW |  |
| "+7 V | 7 | 10,60 | 74,20 | 18,00 | 126,00 |  |
| "+3.0 V | 3 | 4,00 | 12,00 | 6,80 | 20,40 |  |
| "-2 V | 2 | 2,20 | 4,40 | 3,40 | 6,80 |  |
|  |  |  | 90,60 |  | 153,20 |  |
| Driver | WARM |  |  |  |  |  |
| "+7 V | 7 | 9,50 | 66,50 | 15,70 | 109,90 |  |
| "-2 V | 2 | 1,60 | 3,20 | 2,10 | 4,20 |  |
|  |  |  | 69,70 |  | 114,10 |  |
|  |  |  | 160,30 |  | 267,30 |  |
| Boards |  |  |  |  |  | Feedthrough |
|  | PAS = 28x | DRV = 24 x |  |  |  | à xxx mA |
| PSB_1 | 28 | 24 |  |  |  | 208,00 |
| Typ A | WARM | mA | mW | mA | mW | 2,52 |
| "+7 V | 7 | 524,80 | 3673,60 | 880,80 | 6165,60 | 4,23 |
| "+3.0 V | 3 | 112,00 | 336,00 | 190,40 | 571,20 |  |
| "-2 V | 2 | 100,00 | 200,00 | 145,60 | 291,20 | 0,70 |
|  |  |  | 4209,60 |  | 7028,00 |  |
|  | PAS = 22x | DRV = 8 x |  |  |  |  |
| PSB_2_1 | 22 | 8 |  |  |  |  |
| Typ B | WARM | mA | mW | mA | mW | 1,49 |
| "+7 V | 7 | 309,20 | 2164,40 | 521,60 | 3651,20 | 2,51 |
| "+3.0 V | 3 | 88,00 | 264,00 | 149,60 | 448,80 |  |
| "-2 V | 2 | 61,20 | 122,40 | 91,60 | 183,20 | 0,44 |
|  |  |  | 2550,80 |  | 4283,20 |  |
|  | PAS = 30x | DRV =15 x |  |  |  |  |
| PSB_2_2 | 30 | 15 |  |  |  |  |
| Typ C | WARM | mA | mW | mA | mW | 2,21 |
| "+7 V | 7 | 460,50 | 3223,50 | 775,50 | 5428,50 | 3,73 |
| "+3.0 V | 3 | 120,00 | 360,00 | 204,00 | 612,00 |  |
| "-2 V | 2 | 90,00 | 180,00 | 133,50 | 267,00 | 0,64 |
|  |  |  | 3763,50 |  | 6307,50 |  |
|  | PAS = 25x | DRV = 23x |  |  |  |  |
| PSB_3 | 25 | 23 |  |  |  |  |
| Typ D | WARM | mA | mW | mA | mW | 2,32 |
| "+7 V | 7 | 483,50 | 3384,50 | 811,10 | 5677,70 | 3,90 |
| "+3.0 V | 3 | 100,00 | 300,00 | 170,00 | 510,00 |  |
| "-2 V | 2 | 91,80 | 183,60 | 133,30 | 266,60 | 0,64 |
|  |  |  | 3868,10 |  | 6454,30 |  |
|  | PAS = 22x | DRV =20x |  |  |  |  |
| PSB_4 | 22 | 20 |  |  |  |  |
| Typ E | WARM | mA | mW | mA | mW | 2,03 |
| "+7 V | 7 | 423,20 | 2962,40 | 710,00 | 4970,00 | 3,41 |
| "+3.0 V | 3 | 88,00 | 264,00 | 149,60 | 448,80 |  |
| "-2 V | 2 | 80,40 | 160,80 | 116,80 | 233,60 | 0,56 |
|  |  |  | 3387,20 |  | 5652,40 |  |

Table 2

| 03. Sep 98 |  | Power - Consumption BB96B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| PAS | COLD | mA | mW | mA | mW |  |
| "+7 V | 7 | 4,50 | 31,50 | 7,80 | 54,60 |  |
| "+3.0 V | 3 | 5,00 | 15,00 | 9,00 | 27,00 |  |
| "-2 V | 2 | 1,00 | 2,00 | 1,60 | 3,20 |  |
|  |  |  | 48,50 |  | 84,80 |  |
| Driver | COLD |  |  |  |  |  |
| "+7 V | 7 | 4,80 | 33,60 | 6,50 | 45,50 |  |
| "-2 V | 2 | 0,60 | 1,20 | 1,00 | 2,00 |  |
|  |  |  | 34,80 |  | 47,50 |  |
|  |  |  | 83,30 |  | 132,30 |  |
| Boards |  |  |  |  |  | Feedthrough |
|  | PAS = 28x | DRV = 24 x |  |  |  | à xxx mA |
| PSB_1 | 28 | 24 |  |  |  | 208,00 |
| Typ A | COLD | mA | mW | mA | mW | 0,67 |
| "+7 V | 7 | 241,20 | 1688,40 | 374,40 | 2620,80 |  |
| "+3.0 V | 3 | 140,00 | 420,00 | 252,00 | 756,00 | 1,21 |
| "-2 V | 2 | 42,40 | 84,80 | 68,80 | 137,60 |  |
|  |  |  | 2193,20 |  | 3514,40 |  |
|  | PAS = 22x | DRV = 8 x |  |  |  |  |
| PSB_2_1 | 22 | 8 |  |  |  |  |
| Typ B | COLD | mA | mW | mA | mW | 0,53 |
| "+7 V | 7 | 137,40 | 961,80 | 223,60 | 1565,20 |  |
| "+3.0 V | 3 | 110,00 | 330,00 | 198,00 | 594,00 | 0,95 |
| "-2 V | 2 | 26,80 | 53,60 | 43,20 | 86,40 |  |
|  |  |  | 1345,40 |  | 2245,60 |  |
|  | PAS = 30x | DRV =15 x |  |  |  |  |
| PSB_2_2 | 30 | 15 |  |  |  |  |
| Typ C | COLD | mA | mW | mA | mW | 0,72 |
| "+7 V | 7 | 207,00 | 1449,00 | 331,50 | 2320,50 |  |
| "+3.0 V | 3 | 150,00 | 450,00 | 270,00 | 810,00 | 1,30 |
| "-2 V | 2 | 39,00 | 78,00 | 63,00 | 126,00 |  |
|  |  |  | 1977,00 |  | 3256,50 |  |
|  | PAS = 25x | DRV = 23x |  |  |  |  |
| PSB_3 | 25 | 23 |  |  |  |  |
| Typ D | COLD | mA | mW | mA | mW | 0,60 |
| "+7 V | 7 | 222,90 | 1560,30 | 344,50 | 2411,50 |  |
| "+3.0 V | 3 | 125,00 | 375,00 | 225,00 | 675,00 | 1,08 |
| "-2 V | 2 | 38,80 | 77,60 | 63,00 | 126,00 |  |
|  |  |  | 2012,90 |  | 3212,50 |  |
|  | PAS = 22x | DRV =20x |  |  |  |  |
| PSB_4 | 22 | 20 |  |  |  |  |
| Typ E | COLD | mA | mW | mA | mW | 0,53 |
| "+7 V | 7 | 195,00 | 1365,00 | 301,60 | 2111,20 |  |
| "+3.0 V | 3 | 110,00 | 330,00 | 198,00 | 594,00 | 0,95 |
| "-2 V | 2 | 34,00 | 68,00 | 55,20 | 110,40 |  |
|  |  |  | 1763,00 |  | 2815,60 |  |